George Suarez

CSE 310

Homework 4

- Textbook Problems 4.37: Write the HDL gate-level hierarchical description of a four-bit adder-subtractor for unsigned binary numbers. The circuit is similar to Fig.4.13 but without output V. You can instantiate the four-bit full adder described in HDL Example 4.2. (see Problems 4.13 and 4.40.) Write a test bench to test your module.

4bit_adder_subtractor.v

module half_adder(output S, C, input x, y);

// Instantiate primitive gates

xor (S, x, y);

and (C, x, y);

endmodule

module full_adder(output S, C, input x, y, z);

wire S1, C1, C2;

// Instantiate half_adders

half_adder HA1 (S1, C1, x, y);

half_adder HA2 (S, C2, S1, z);

or G1 (C, C2, C1);

endmodule

module ripple_carry_4_bit_adder(output [3:0] Sum, output C4,

input [3:0] A, B, input C0);

wire C1, C2, C3;    // Intermediate Carries

// Instantiate full_adders

full_adder FA0 (Sum[0], C1, A[0], B[0], C0),

FA1 (Sum[1], C2, A[1], B[1], C1),

FA2 (Sum[2], C3, A[2], B[2], C2),

FA3 (Sum[3], C4, A[3], B[3], C3);

endmodule

4bit_adder_subtractor_tb.v:

module four_bit_adder_subtractor_tb;

// Inputs

reg [3:0] A;

reg [3:0] B;

reg Cin;

// Outputs

wire Cout;

wire [3:0] Sum;

// Instantiate the Unit Under Test (UUT)

ripple_carry_4_bit_adder uut(.Sum(Sum), .C4(Cout), .A(A), .B(B), .C0(Cin));

initial

begin

$display("Time\t A \t B \t Cin \t | Sum \t Cout");

$monitor("%3g\t %b \t %b \t %b \t | %b   %b", $time, A, B, Cin, Sum, Cout);

end

initial

begin

A = 0000; B = 0001; Cin = 0;

#10 A=0100; B=1011; Cin=0;

#10 A=1000; B=0010; Cin=0;

#10 A=0100; B=0011; Cin=1;

#10 A=1100; B=0011; Cin=1;

#10 A=1100; B=0100; Cin=1;

#10 A=0100; B=0000; Cin=0;

#10 A=0100; B=0001; Cin=0;

#10 A=0100; B=0010; Cin=0;

#10 A=0100; B=0000; Cin=1;

#10 A=0100; B=0001; Cin=1;

#10 A=0100; B=0010; Cin=1;

#10 $finish;

end

endmodule

Output:

Time	 A 	 	 B 	     Cin  |Sum   Cout

0	 0000 	 0001 	 0 	 | 0001   0

10	 0100 	 0011 	 0 	 | 0111   0

20	 1000 	 1010 	 0 	 | 0010   1

30	 0100 	 1011 	 1 	 | 0000   1

40	 1100 	 1011 	 1 	 | 1000   1

50	 1100 	 0100 	 1 	 | 0001   1

60	 0100 	 0000 	 0 	 | 0100   0

70	 0100 	 0001 	 0 	 | 0101   0

80	 0100 	 1010 	 0 	 | 1110   0

90	 0100 	 0000 	 1 	 | 0101   0

100	 0100 	 0001 	 1 	 | 0110   0

- 0100 	 1010 	 1 	 | 1111   0

- Textbook Problems 4.13 (p.184):

| Cin = 0 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
|  | 1 | 1 | 0 | 0 |
| A | 0 | 1 | 1 | 1 |
| B | 0 | 1 | 1 | 0 |
| A + B = Cout = 0 | 1 | 1 | 0 | 1 |

Sum = 1101

C = Cout = 0

V = (Cout ^ C3) = 0 ^ 1 = 1

| Cin = 0 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 |
| A | 1 | 0 | 0 | 0 |
| B | 1 | 0 | 0 | 1 |
| A + B = Cout = 1 | 0 | 0 | 0 | 1 |

Sum = 0001

C = Cout = 1

V = (Cout ^ C3) = (1 ^ 0) = 1

| Cin = 1 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
|  | 1 | 1 | 1 | 1 |
| A | 1 | 1 | 0 | 0 |
| B | 0 | 1 | 1 | 1 |
| A + B = Cout = 1 | 0 | 1 | 0 | 0 |

Sum = 0111

C = Cout = 1

V = (Cout ^ C3) = (1 ^ 1) = 0

| Cin = 1 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
|  | 1 | 0 | 1 | 1 |
| A | 0 | 1 | 0 | 1 |
| B | 0 | 1 | 0 | 1 |
| A + B = Cout = 0 | 1 | 0 | 1 | 1 |

Sum = 1011

C = Cout = 0

V = (Cout ^ C3) = (0 ^ 1) = 1

| Cin = 1 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 |
| A | 0 | 0 | 0 | 0 |
| B | 0 | 0 | 0 | 1 |
| A + B = Cout = 0 | 1 | 1 | 1 | 1 |

Sum = 1111

C = Cout = 0

V = (Cout ^ C3) = (0 ^ 0) = 0

- Textbook Problems 4.44:

ALU.v:

module ALU ( output [7:0] Y, input[7:0] A, B, input [2:0] Sel);

always @ (A, B, Sel)

begin

y = 0;

case (Sel)

3'b000: y = 8'b0;

3'b001: y = A & B;

3'b010: y = A | B;

3'b011: y = A ^ B;

3'b100: y = A + B;

3'b101: y = A - B;

3'b110: y = ~A;

3'b111: y = 8'hFF;

endcase

end

endmodule

~

- Design a binary sequence detector that detects the sequence 000. Overlap is allowed. You may use either D flip-flops or JK flip-flops. Write a Verilog program to verify your design.

- State Diagram

(ii) Assign States

Q1, Q0

(iii) State Table

| Q1(t) | Q0(t) | I(t) | Q1(t+1) | Q0(t+1) | F(t) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 |

Using D flip-flops:

D0=Q1I+Q0I

|  | Q1Q2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I |  | 00 | 01 | 11 | 10 |
|  | 0 | 0 | 0 | X | 1 |
|  | 1 | 0 | 1 | X | 0 |

D1=Q1*Q0*I+Q1I

|  | Q1Q2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I |  | 00 | 01 | 11 | 10 |
|  | 0 | 1 | 0 | X | 0 |
|  | 1 | 0 | 0 | X | 1 |

(iv) Circuit Diagram:

Self-Evaluation: I have answered every question in this homework assignment to the best of my ability. I would give myself the maximum points allowed for this assignment which is 55 points.